# SECTION 260936

# MODULAR DIMMING CONTROLS

- - GENERAL

  - SUMMARY

    - Section Includes:

      - Lighting Control Devices

    - Related Sections:

      - Section 122000 - Window Treatments: Window treatments controlled by modular dimming control system.

      - Section 262726 - Wiring Devices

      - Section 265113 – Interior Lighting Fixtures, Lamps, and Ballasts: Fluorescent lighting ballasts controlled by central dimming control system.

      - Section 260923 – Lighting Control Devices: Occupancy sensors used in conjunction with modular dimming control system.

  - REFERENCES

    - American National Standards Institute/Institute of Electrical and Electronic Engineers (ANSI/IEEE):

      - C62.41-1991 – Recommended Practice for Surge Voltages in Low-Voltage AC Power Circuits.

    - ASTM International (ASTM):

      - D4674 -02a Standard Test Method for Accelerated Testing for Color Stability of Plastics Exposed to Indoor Fluorescent Lighting and Window-Filtered Daylight.

    - International Organization for Standardization (ISO):

      - 9001:2000 – Quality Management Systems.

    - National Electrical Manufacturers Association (NEMA):

      - WD1 (R2005) - General Color Requirements for Wiring Devices.

    - Underwriters Laboratories, Inc. (UL):

      - 244A – Solid State Appliance Controls.

      - 1472 (1996) - Solid-State Dimming Controls.

  - SYSTEM DESCRIPTION

    - Radio Frequency Light Control System (RFLCS)

      - Modular dimming and switching components.

      - Two-way Radio Frequency controlled wall-mounted dimmers and switches, lamp socket module dimmers and switches, master controls, and interfaces without the use of a central processor.

  - SUBMITTALS

    - Submit under provisions of Section 013300.

    - Specification Conformance Document:  Indicate whether the submitted equipment:

      - Meets specification exactly as stated.

      - Meets specification via an alternate means and indicate the specific methodology used.

    - Shop Drawings; include:

      - Load schedule indicating actual connected load, load type, and control zones.

      - Schematic of system.

    - Product Data: Catalog cut sheets with performance specifications demonstrating compliance with specified requirements.

  - CLOSEOUT SUBMITTALS

    - Sustainable Design Closeout Documentation (LSC-LEED-DOC)

      - Lighting Control System Manufacturer to provide Enhanced Start-up documentation that details the start-up procedure being performed including a process to follow, details on tests performed and an area that documents any test results.

  - QUALITY ASSURANCE

    - Manufacturer: Minimum 10 years of experience in manufacture of architectural lighting controls and Radio Frequency Light Control products.

    - Manufacturer’s Quality System: Registered to ISO 9001:2000 Quality Standards, including in-house engineering for product design activities.

    - Radio Frequency Light Control System (RFLCS):

      - Listed by UL specifically for the required loads. Provide evidence of compliance upon request.

      - Approval of local radio communications governing agency. Provide evidence of compliance upon request.

  - PROJECT CONDITIONS

    - Do not install equipment until following conditions can be maintained in spaces to receive equipment:

      - Ambient temperature: 0 degrees to 40 degrees C (32 degrees to 104 degrees F).

      - Relative humidity: Maximum 90 percent, non-condensing.

      - Light control system must be protected from dust during installation.

  - WARRANTY

    - Provide Manufacturer’s Warranty:

      - Standard 2-year warranty, Includes:

        - 100 Percent Replacement Parts for Manufacturer Lighting System Components

        - 100 Percent Manufacturer Labor Coverage to Troubleshoot and Diagnose a Lighting Issue

        - First –Available Onsite or Remote Response Time

        - 24 Hours Per Day, 7 Days Per Week Telephone Technical Support, Excluding Manufacturer Holidays

        - Remote Diagnostics for Applicable Systems

        - 4-Hours of Remote

      - Enhanced 8-year Silver Level limited parts warranty, Includes:

        - Years 1-2:

          - 100 Percent Replacement Parts for Manufacturer Lighting System Components

          - 100 Percent Manufacturer Labor Coverage to Troubleshoot and Diagnose a Lighting Issue

          - First-Available Onsite or Remote Response Time

          - Remote Diagnostics for Applicable Systems

          - 4-Hours of Remote Programming for Applicable Systems

        - Years 3-5:  50% Replacement Parts Coverage

        - Years 6-8:  25% Replacement Parts Coverage

        - 24 Hours Per Day, 7 Days Per Week Telephone Technical Support, Excluding Manufacturer Holidays

      - Enhanced 8-year Gold Level limited parts warranty, Includes:

        - Years 1-2:

          - 100 Percent Replacement Parts for Manufacturer Lighting System Components

          - 100 Percent Manufacturer Labor Coverage to Troubleshoot and Diagnose a Lighting Issue

          - 72-Hour Onsite or Remote Response Time

          - Annual Scheduled Preventative Maintenance Visit

          - Remote Diagnostics for Applicable Systems

          - 4-Hours of Remote Programming for Applicable Systems

        - Years 3-5:  50% Replacement Parts Coverage

        - Years 6-8:  25% Replacement Parts Coverage

        - 24 Hours Per Day, 7 Days Per Week Telephone Technical Support, Excluding Manufacturer Holidays

      - Enhanced 8-year Platinum Level limited parts warranty, Includes:

        - Years 1-2:

          - 100 Percent Replacement Parts for Manufacturer Lighting System Components

          - 100 Percent Manufacturer Coverage to Troubleshoot and Diagnose a Lighting Issue

          - 24-Hour Onsite or Remote Response Time

          - Annual Scheduled Preventative Maintenance Visit

          - Remote Diagnostics for Applicable Systems

          - 4-Hours of Remote Programming for Applicable Systems

        - Years 3-5:  50% Replacement Parts Coverage

        - Years 6-8:  25% Replacement Parts Coverage

        - 24 Hours Per Day, 7 Days Per Week Telephone Technical Support, Excluding Manufacturer Holidays

    - Provide Manufacturer’s Support and Maintenance Plan for 8 years covering 100 percent parts and 100 percent labor and additional benefits as described below beginning 2 years after system startup completion.]

      - Silver Level Support and Maintenance Plan, Includes:

        - 100 Percent Parts for Manufacturer Lighting System Components

        - 100 Percent Manufacturer Labor Coverage for Troubleshooting and Diagnosis of Lighting Issues

        - 24 Hours Per Day, 7 Days Per Week Telephone Technical Support, Excluding Manufacturer Holidays

        - First-Available Onsite or Remote Response Time

        - Remote Diagnostics for Applicable Systems

        - 4-Hours of Remote Programming for Applicable Systems

      - Gold Level Support and Maintenance Plan, Includes:

        - 100 Percent Parts for Manufacturer Lighting System Components

        - 100 Percent Manufacturer Labor Coverage for Troubleshooting and Diagnosis of Lighting Issues

        - 24 Hours Per Day, 7 Days Per Week Telephone Technical Support, Excluding Manufacturer Holidays

        - 72-Hour On-Site or Remote Response Time

        - Annual Scheduled Preventative Maintenance Visit

        - Remote Diagnostics for Applicable Systems

        - 4-Hours Remote Programming for Applicable Systems

        - Includes Service Coverage Upgrade of the Initial 2-year 100 Percent Manufacturer Labor Coverage to Gold Level Coverage.

      - Platinum Level Support and Maintenance Plan, Includes:

        - 100 Percent Parts for Manufacturer Lighting System Components

        - 100 Percent Manufacturer Labor Coverage for Troubleshooting and Diagnosis of Lighting Issues

        - 24 Hours Per Day, 7 Days per Week Telephone Technical Support, excluding Manufacturer Holidays

        - 24-Hour On-Site or Remote Response Time

        - Annual Scheduled Preventative Maintenance Visit

        - Remote Diagnostics for Applicable Systems

        - 4-Hours Remote Programming for Applicable Systems

        - Includes Service Coverage Upgrade of the Initial 2-Year 100 Percent Manufacturer Labor Coverage to Platinum Level Coverage.

    - For ballasts, see section 265113 – Interior Lighting Fixtures, Lamps, and Ballasts: Fluorescent Electronic Dimming Ballasts.

    - For Sivoia shades, see section 122413 – Roller Window Shades.

  - MAINTENANCE material submittals

    - Make ordering of new equipment for expansions, replacements, and spare parts available to end user.

    - Make new replacement parts available for minimum of ten years from date of manufacture.

- - PRODUCTS

  - MANUFACTURERS

    - Basis of design product: Lutron Stanza Radio Frequency Light Control System or subject to compliance and prior approval with specified requirements of this section, one of the following:]

      - <Insert manufacturer’s name>

    - Substitutions: Under provisions of Division 1.

      - All proposed substitutions (clearly delineated as such) must be submitted in writing for approval by the design professional a minimum of 10 working days prior to the bid date and must be made available to all bidders.

      - Proposed substitutes must be accompanied by a review of the specification noting compliance on a line-by-line basis.

      - By using pre-approved substitutions, the contractor accepts responsibility and associated costs for all required modifications to circuitry, devices, and wiring.

      - Provide complete engineered shop drawings (including power wiring) with deviations for the original design highlighted in an alternate color to the engineer for review and approval prior to rough-in.

  - GENERAL

    - Provide system that is designed, tested, manufactured, and warranted by a single manufacturer.

    - Radio Frequency communications to operate within 434 MHz band, free from interference from other building and system frequencies.

    - Light Controls: Ten-year operational life while operating continually at any temperature in an ambient temperature range of 0 degrees C (32 degrees F) to 40 degrees C (104 degrees F) and 90 percent non-condensing relative humidity.

    - Designed and tested to withstand electrostatic discharges up to 15,000 V without impairment per IEC 801-2.

    - Electrolytic capacitors to operate at least 20 degrees C below the component manufacturer's maximum temperature rating when device is under fully-loaded conditions in 40 degrees C (104 degrees F) ambient temperature.

    - Design and test line voltage devices to withstand line-side surges without impairment to performance.

      - Power handling devices: Withstand surges without impairment of performance when subjected to surges of 6,000 volts, 200 amps per ANSI/IEEE C62.41C.

    - Power failure memory such that if power is interrupted and subsequently returned, lights will automatically return to same levels (dimmed setting, full on, or off) prior to power interruption within 3 seconds.

      - All programmable information, including system identification codes, addresses, and master control button programming to be retained in non-volatile memory and to remain intact in the case of a 10 year power loss to the system.

    - Color:

      - Match NEMA WD1, Section 2.

      - Color variation in same product family: Maximum ΔE=1, CIE L*a*b color units.

      - Visible parts: Exhibit ultraviolet color stability when tested with multiple actinic light sources as defined in ASTM D4674. Provide proof of testing upon request.

    - Provide faceplates with concealed mounting hardware.

    - Engrave keypad buttons with appropriate zone or scene engraving descriptions.

    - All text, borders, logos, and graduations to use graphic process that chemically bonds markings to faceplate or buttons, resistant to removal by scratching and cleaning.

  - PERMANENTLY MOUNTED DIMMERS;

    - Product: Lutron Model: SZ-6D-color or SZ-6ND-color

    - Description:

      - Provide local control:

        - To override master controls.

        - In the event that communication with other devices in the system is interrupted.

      - Capable of controlling MLV and Incandescent loads without a neutral wire

      - Capable of controlling interfaces to other loads when used with a neutral wire

      - Capable of providing smooth continuous dimming.

      - Operate and control lamps without integrated outlets when used in conjunction with an outlet approved by UL for dimming.

      - Operate and control dimmable compact fluorescent lamps (CFLs).

    - Load Handling Thyristors (SCRs and triacs), Field Effect Transistors (FETs), and Isolated Gate Bipolar Transistors (IGBTs): Manufacturer’s maximum current rating minimum two times control’s rated operating current.

    - Capable of withstanding repetitive inrush current of 50 times operating current without impacting lifetime of dimmer/switch.

    - Utilize air gap off for permanently-mounted dimmers/switches to disconnect the load from line supply.

    - Dimmers to meet following load-specific requirements:

      - Magnetic Low Voltage (MLV) transformer:

        - Contain circuitry designed to control and provide a symmetrical AC waveform to input of magnetic low voltage transformers per UL 1472, Section 5.11.

        - Dimmers using unipolar load current devices (such as FETs or SCRs) to include DC current protection in the event of a single device failure.

      - Fluorescent electronic dimming ballast: Refer to Section 265113 for dimming ballast specifications and performance.

  - LAMP SOCKET DIMMERS

    - Product: Lutron Model: SZ-1SD-WH

    - Description:

      - Provide local control at lamp socket:

        - To override master controls.

        - In the event that communication with other devices in the system is interrupted.

      - Capable of dimming incandescent, halogen, and dimmable compact fluorescent bulbs

      - Capable of switching non-dimmable compact fluorescent bulbs without swapping device

      - Capable of providing smooth continuous dimming.

      - Operate and control lamps with integrated outlets

      - Operate and control dimmable compact fluorescent lamps (CFLs).

    - Load Handling Thyristors (SCRs and triacs), Field Effect Transistors (FETs), and Isolated Gate Bipolar Transistors (IGBTs): Manufacturer’s maximum current rating minimum two times control’s rated operating current.

    - Capable of withstanding repetitive inrush current of 50 times operating current without impacting lifetime of dimmer/switch.

  - SWITCHES

    - Product: Lutron Model: SZ-6ANS

    - Description:

      - Provide local control:

        - To override master controls.

        - In the event that communication with other devices in the system is interrupted.

      - Capable of controlling incandescent, fluorescent, magnetic low-voltage, electronic low-voltage, and high intensity discharge loads with a neutral wire

    - Load Handling Thyristors (SCRs and triacs), Field Effect Transistors (FETs), and Isolated Gate Bipolar Transistors (IGBTs): Manufacturer’s maximum current rating minimum two times control’s rated operating current.

    - Capable of withstanding repetitive inrush current of 50 times operating current without impacting lifetime of dimmer/switch.

    - Utilize air gap off to disconnect the load from line supply.

  - KEYPADS

    - Product: Lutron Model: SZ-2B-color, SZ-3B-color, SZ-4B-color, SZ-5B-color or SZ-3/1B-color

    - Functionality:

      - Control any single lighting load or combination of lighting loads, which are part of the RFLCS.

      - LEDs on each button to reflect the true system status.

      - Master control keypad to be field programmable.

  - Card switch guest room control

    - Product: Lutron Model SZ-CS-CCO

    - Functionality:

      - Provide maintained contact closure output when card key is inserted to turn lights on.

      - Turns lights off when card key is removed.

    - Accept standard size card keys: 54mm x 86 mm x 0.8 mm (2.125 inches x 3.375 inches x 0.03 inches)

    - Provide flashing backlit slider when card key is not present.

  - POWER INTERFACES

    - Product: PHPM-PA or GRX-TVI

    - Electrical:

      - Phase independent of control input.

    - Diagnostics and Service: Replacing power interface does not require re-programming of system or devices.

  - CONTROL INTERFACES

    - Contact Closure Interface; Lutron Model SZ-IO

      - The contact closure inputs will accept both momentary and maintained contact closures.

      - The contact closure outputs will provide “dry” closure outputs.

    - Serial / Ethernet Interface; Lutron Model SZ-CI-PRG

      - The Serial / Ethernet interface must be used to change system programming

      - The Serial / Ethernet interface allows integration of the RFLCS to other room or building systems

  - SOURCE QUALITY CONTROL

    - Perform full-function testing on all completed assemblies at end of line. Statistical sampling is not acceptable.

- - EXECUTION

  - INSTALLATION

    - Install equipment in accordance with manufacturer’s installation instructions.

    - Provide complete installation of system in accordance with Contract Documents.

    - Define each dimmer's load type, assign each load to a zone, and set control functions.

    - Provide equipment at locations and in quantities indicated on Drawings. Provide any additional equipment required to provide control intent.

  - STARTUP AND PROGRAMMING

    - Provide factory-certified field service engineer to a site visit to ensure proper system installation and operation under following parameters:

      - Qualifications for factory-certified field service engineer:

        - Minimum experience of 2 years training in the electrical/electronic field.

        - Certified by the equipment manufacturer on the system installed.

      - Make a visit upon completion of installation of RFLCS control system:

        - Verify connection and location of controls.

        - Verify system operation control by control, zone by zone.

        - Verify proper Radio Frequency communication.

        - Verify proper integration of manufacturers interfacing equipment.

        - Provide programming.

        - Obtain sign-off on system functions.

        - Train owner on system operation.

END OF SECTION